GHANA TERTIARY EDUCATION COMMISSION

STANDARDS FOR PHYSICAL FACILITIES

FOR

TERTIARY INSTITUTIONS

IN

GHANA

REVISED NOVEMBER 2014

STANDARDS FOR PHYSICAL FACILITIES

PART 1 – PRELIMINARY

These Standards may be cited as the Physical Facilities Standards.

Introduction:

In order to achieve the desired objectives of teaching/learning and ultimately adding to the Body of Knowledge, it is imperative that teaching/learning activities take place in purpose-designed or purposely-modified structures for education.  These standards capture both best practice and the minimum standards needed for tertiary institutions. The standards should be read in conjunction with the National Building Regulations 1996 LI1630, the Ghana National Fire Service Act 537 of 1997 – Fire Regulations LI 1724and the Disability Act 715 of 2006

Definitions

- In these Standards, unless the context otherwise requires –

“tertiary institution” includes, University, Polytechnic and all other institutions regulated by the GTEC

“a recognised water laboratory” means a laboratory registered with or belonging to the Ministry of Health or Ministry of Works and Housing, or any other public institution, person or organisation duly authorised by the Government  to undertake water analysis;

“adequate”  means sufficient  in relation to any set guidelines or requirements;

“adequate water supply” means water in such quantities as would permit maintenance of a reasonable level of  personal and environmental hygiene;

“Auditorium” means a large room where students/public may sit and listen to a lecture or watch a performance or event with clear sight lines.

“Building Code” means Building codes issued by Ministries of Works and Housing and Local Government;

“Classroom” means a room accommodating several students (between 30 to 100) for the purposes of instruction;

“Departmental areas “means offices occupied by teaching, administrative and secretarial staff of a teaching department of a tertiary institution;

“Full time student equivalent” (FTSE) means the number of hours of teaching required by one student to fulfill the requirements of the course in an academic year;

“Lecture Room” means a room for teaching/learning accommodating more than 100 seated students with a facility for writing/taking down notes.

“Master plan” means a programme of a tertiary institution showing the order of development of physical and academic aspects for a given period of time;

“Physical facilities” means any structure fixed or movable or of whatever kind and any part thereof used or intended to be used for teaching, lecturing or instructing or as a dwelling house or for recreational and all ancillary facilities thereto including drainage works, services installations and road works;

“Reader station” means space occupied by a student in a library;

“Relevant local authority” means a city council, metropolitan assembly, municipal council or such local authority under whose jurisdiction a tertiary institution is situated;

“Research” means an investigative study of any physical, biological or social phenomenon through a systematic approach;

“Research area” means a facility where research is carried out;

“Residential tertiary institution” means a tertiary institution in which ten or more students are boarded.

“Safe water supply” means a supply of water free from any contamination;

“Seminar room” refers to a room for interacting/teaching small groups of up to 30 students

“Sewage” means spent water emanating from toilets;

“Student” means a person enrolled to pursue a degree, diploma or certificate course;

“Site works” means a place where construction activities are carried out;

“Surface water” means any ground water, subsoil water, storm water or rain water which discharges on the ground surface;

“Utility services” means any service that is connected to any building or living space through pipe or wire network for helping to convey fluids, energy and messages, in and out of any building or living space;

“Waste water”  means any water after it has been fouled by a variety of uses and which constitutes a combination of liquid and water carried wastes, emanating from residences, toilets, kitchens, laundries, laboratories and workshops.

- These Standards shall apply to-

- all buildings or part of buildings purposely designed and constructed for tertiary institution use;

- any alterations and extensions of tertiary institution buildings or parts of the buildings;

- all buildings undergoing material change of use into tertiary institution buildings; and

- all related utility services installations, and site works for a tertiary institution.

- All institutions regulated by the GHANA TERTIARY EDUCATION COMMISSION

- These Standards are prescribed in respect of the following factors –

- Ministry of Health –

- application of by-laws, the National Building Regulations LI 1630 and the Ministry of Health Act;

- materials used in the buildings;

- building lighting, ventilation and sound proofing;

- building hygiene and sanitation;

- water supply and waste disposal.

- Public safety

- structural soundness and stability;

- fire safety – fire resistance, fire protection and safety precaution, firefighting and fire escape.

- Physical facilities –

- building facilities including classrooms and lecture theatres, staff offices, research areas, seminar rooms, laboratories, workshops studios, clinics, libraries, students’ residences, staff residences, communal and social services areas such as worship, kitchen, dining, common rooms, recreational facilities.

- utility and other services including water supply, power, telephone, external drainage and waste water services, access roads, parking and appropriate landscaping;

- land including requirements and location ownership and tenure.

- Environmental Impact Assessment (including Traffic Impact Assessment).

- Spatial requirements

- the minimum size and floor areas for various buildings;

- the minimum unit numbers of various types of buildings;

- the locational relationship of various facilities.

PART II – MINIMUM STANDARDS

- Every tertiary institution shall provide the following physical facilities to accommodate its activities –

- Classrooms, lecture rooms, seminar rooms

- auditorium or lecture theatre

- Academic and non-academic offices;

- library /information resource centre (including E Learning Facilities)/access to sources of information;

- the information access facility may include a knowledge exchange/socialization  area

- student areas for knowledge exchange/socialization   with  recreation facilities;

- office for Students’ Representative Council

- outdoor recreation facilities in form of games or sports facilities;

- proper drainage system, proper sanitation and water supply;

- Infirmary/sick bay.

- Adequate Parking space for staff and students

- Every residential tertiary institution, shall, in addition to the facilities to be provided under paragraph (4) provide the following physical facilities which meet the minimum national safety standards–

- Provision should be made for cafeteria services, kitchen and dining facilities for student accommodation, including adequate laundry and storage facilities.

- Adequate parking space for students.

- students’ common room

- Every building used or intended to be used as part of the tertiary institution physical facility shall comply with the requirements of the National Building Regulations LI 1631 of 1996 and provisions of the Ministry of Health Act, in particular –

- The area for the buildings must be approved by the appropriate authorities

- the minimum  requirements for health and safety of the public to which the premises of every tertiary institution shall conform, shall be in line with those prescribed by the National Building Regulations LI 1630 of 1996 or any amendments thereof, the Ministry of Health Act and the requirements prescribed in the Schedule;

- any building designed and constructed for use as tertiary institution building or any building altered so as to be used as a tertiary institution building or any building which has undergone material change of use into tertiary institution building must be approved by the relevant local authority;

- all buildings and other physical facilities used by a tertiary institution should show evidence of approval by the appropriate authorities and in addition show evidence of the ff–

- drawings of the proposed building(s);

- building permit for development of  new structures;

- structural integrity test confirming compliance to minimum required structural strength;

- alterations or extensions if any for existing buildings; or

- proposed change of use if any; and

- a certificate of occupation for the newly constructed or altered buildings.

PART III – PUBLIC SAFETY

- Every tertiary institution shall operate in physical facilities that are safe for the public and in particular –

- all buildings and other physical facilities used by a tertiary institution to accommodate tertiary institution activities shall be functional and serving the purpose for which it is designated;

- the buildings shall be kept in a good state of repair and maintenance and shall be free from structural failures, (for example deflection, spalling, cracking or dilapidation of building material fabric and components).

- The ambience must be congenial to academic activities i.e adjoining properties and activities must be complementary to academic activities and must not be the source of noise or other activities that will disturb and make it difficult for teaching and learning to take place.

- Every tertiary institution which erects or causes to be erected a building or any other structure intended to accommodate tertiary institution activities shall employ, for the purposes of architectural design, a registered architect and for the purposes of structural design, thereof, a registered structural engineer, and shall retain the services of such architect or structural engineer for the purpose of supervising the erection of such building and in particular –

- on completion of construction of such building the architect or structural engineer shall provide a certificate that the work or works have been carried out in accordance with the design and that it complies in all respects with the provisions of the National Building Regulations and the appropriate code of practice;

- all drawings in respect of the structural framework of steel buildings, reinforced concrete buildings or structural timber buildings and calculations connected therewith, shall be signed by the registered structural engineer responsible for their design;

- The standards specified in paragraph 8 shall apply to all categories of buildings designated for tertiary education purposes/institutions –

- 1)	All buildings and other physical facilities used by a tertiary institution to

accommodate tertiary institution activities shall provide adequate fire safety equipment certified by the GNFS.

2)	All buildings and other physical facilities shall conform to the requirements of the National Building Regulations, the Ministry of Health Act and other standard practices with regard to fire resistance, means of fire escape, access from fire escape and firefighting equipment.

3)	All means of fire escape shall be properly designated with appropriate signs and symbols, maintained and kept free from any obstruction.

4)	All institutions shall carry out at least once a year, an induction of all staff and students on what to do in case of a fire or earthquake. This shall be carried out and certified by the GNFS.

5)	The guidelines specified in the National Building Regulations LI 1630 of 1996 shall apply.

- Fire detection and prevention

  - In every building used by a tertiary institution as a classroom, lecture theatre, seminar room, auditorium, laboratory, library, dormitory or residential hall, there shall be provided adequate firefighting equipment such as hose reels, portable fire extinguishers, fire buckets, dry risers, wet risers, fire hydrants, sprinklers, and water storage tanks.

  - Where a tertiary institution is located outside the area covered by Government or local authority fire station, the institution concerned shall seek the assistance/advice of the GNFS.

  - Every tertiary institution shall be provided with a dedicated water supply system for firefighting; and the water supply needs for emergency firefighting purposes shall be drawn through the main water supply distribution system via a series of underground hydrants located at regular intervals throughout the site of the tertiary institution.

PART IV – ACADEMIC BUILDING FACILITIES

The classroom is said to be the heart of the academic environment. The classroom design and layout are key in assisting the teacher/lecturer/instructor to attain the objectives of imparting knowledge or the techniques of acquiring knowledge to the student.  The guiding principles are to:

- Allow everyone to be seen and heard

- Take advantage of new technology for teaching and learning

- Support the dynamic presentation of information

- Design for mentoring and apprenticeship

- Design for comfort within the class room

- Every tertiary institution shall provide classrooms which are adequate in number and shall be of adequate size; and in establishing the adequacy of classrooms, the minimum floor area per student specified in Table 1 shall be provided.

Table 1: Minimum Classroom Floor Area per Student

No. of Students	Space in square metres		Space in square metres

with desk and chair			with chairs only

0-29				2.25					1.9

30-39				1.9					1.4

40-59				1.7					1.3

60-69				1.7					1.2

100-149			1.7					1.0

150-299			1.5					0.9

300- 500			1.4					0.9

Above 500			1.35					0.9

- The tertiary institution shall have enough classrooms for every course, provided that there shall be a minimum of three classrooms to each four year course of study.

- Doors must open out to the corridor but must be located so that they do not block corridor traffic. A recessed entrance may suffice.

- Doors should have a narrow vision panel that can allow visibility into the room without the need to disrupt the class.

- A minimum of two doors is required for a class room. There must be at least one door to the rear of the classroom so as to minimize interruptions by students entering the classroom late or leaving early. Another door must be near the front to facilitate accessibility for the physically challenged.

- Classroom Proportions: All seats must be located within a 90 degree viewing angle from the center of the projection screen. That is, within 45 degree horizontal angles from the perpendicular to the center of screens. Classrooms should be narrow enough to permit all seats to be within the 90 degree viewing angle from the front wall, but no narrower. Rooms that are too narrow and deep make it hard for students and instructors to interact.

- The distance from the front wall to the first row of seats should be 1-2 times the height of the projection screen. Typical screen height in a flat-floor classroom is 2.5 m, so the first row of seats must be minimum 2.5m from the front wall; 2.6m to 2.9m is preferred.

- Where a multimedia lectern is used it must be placed to one side of the front teaching wall, leaving students an unobstructed view of the writing surface and projection screen. Teaching and learning equipment should be installed either in the lectern or in a rack near the lectern.

- Every room used as a classroom shall be of sound construction and shall be provided with windows the effective area of which shall not be less than 20 per cent of the floor area of the room and 75 per cent of the window area shall be openable to the external air.

- No room used as classroom shall have a height of less than 2.60 metres from floor to ceiling, or where there is no ceiling, to the wall plate.

- All walls must extend to the floor above or to the roof construction, and not stop at ceiling.

- Walls between classrooms will have a Sound Transmission Class (STC) rating of at least 50. Walls separating classrooms from common spaces or restrooms must have an STC of at least 53. Walls separating classrooms from mechanical spaces or other areas with high noise levels must have at least an STC of 60. The floors slab and walls must be isolated in order to prevent the transmission of vibrations.

- Ambient noise level should not exceed 35 decibels when measured with the A-scale of a sound level meter.

- Voice amplification is required for rooms seating 100 and/or which is larger than 170 square metres.

- Every room used as a classroom shall be provided with adequate, well illuminated writing surface (of between 300 to 500 lux) at each seat and a place to set books, papers and or other equipment.

- All classrooms may normally have no less than two separately controlled lighting areas – seating area and instructional area. The ability to dim both areas shall be provided as standard. When the classroom is dimmed for projection, some lighting will be required at the presentation area. Special lighting on the equipment rack or technology controls may be needed.

- In auditoria seating over 100, a separate light for a sign language interpreter will need to be placed adjacent to the front of the room.

- Every tertiary institution shall be provided with adequate room to accommodate academic and non-academic staff offices and seminar rooms and in determining the adequacy of departmental areas, the following factors shall be taken into consideration: -

- the total usable space provided for use as departmental areas shall be not less than those specified in the following Table 2-

Table 2- Minimum Departments’ Useable Areas per Student

Course							*Usable space in square

metres per Student

Arts and Humanities based courses 	..	..		3.0

Biological and Physical Sciences based courses	..		9.0

Human and Veterinary Medicine and related courses		15.0

Engineering, Surveying and related courses	..		10.0

Architecture, Planning, Building Technology, Design and

related courses					..		10.0

Social Science based courses			..		3.0

*The usable space per student is calculated by dividing the total area of the department by the number of students in the department.

The minimum sitting area per student shall not be less than 0.83m2

- the departmental offices shall comprise of the following minimum number of offices

- one departmental office

- one head of department office

- one secretary’s office

- one general office for a staff secretaries

- one office for every member of academic staff

- academic and non-academic member of staff offices shall have the following minimum floor areas –

- head of department and professor; 18 square metres,

- academic staff: 9 square metres

- non-academic staff: 6 square metres

- A tertiary institution shall provide adequate central administration offices to accommodate the head administrator and support staff of the tertiary institution and each constituent college; and in assessing the adequacy of administration offices the following factors shall be taken into account –

- the total office space provided for use by administrative staff shall not be less than 0.55 square metre of usable office space for every full time student equivalent enrolled, provided the minimum total office space provided for the tertiary institution’s central administrative is 50 square metres;

- no room used as an administrative office shall be less than six (6) square metres in area;

- the administrative offices shall be conveniently accessible with visible and adequate directional signs.

The Library (Access to sources of information)

Two major types of libraries: electronic and the physical. Standards will be defined for each by the Library Expert.

If the classroom is said to be the heart of higher education, then the library is its soul. Libraries like classrooms, must also reflect the dynamic nature of tertiary education. Though students use Kindles, I Pads, Tablets and other media that allow them instant access to information, the library still holds an attraction because it is:

  - a convenient spot between classes

  - where to collaborate on group work

  - close to many resources

  - a safe, non-distracting place to study

  - the place for computing software, copying, printing, scanning

  - a great atmosphere

Contemporary libraries must:

- Provide students access to electronic libraries

- Support social learning

- Support the librarian’s evolving role

- Optimize the performance of informal spaces

- Plan for adjacencies

- Provide for individual comfort, concentration, and security

- Provide spaces that improve awareness of, and access to the library’s resources,

- Every tertiary institution shall provide a well-planned, secure and sufficient space to house the library’s collections, provide adequate workspace for staff and conducive study space in numbers and variety to meet the needs of users; especially the emerging specialty space such as tutoring rooms, writing centres, group study rooms.

- the library shall conform to the requirements of the National Building Regulations;

- The library shall be flexible in layout, allocated floor space and technical infrastructure to allow reasonable adaptation to rapid change.

- the library shall be designed to offer 24 hours access to specific arears such as computer laboratory, open reserve collection, photocopying and study facilities

- the library shall produce a written disaster plan for fire and flood control.

- the library shall have sufficient doorways to encourage rapid exit in case of fire or other emergency.

- the building shall have adequate natural lighting and ventilation.

- the library shall be friendly and comfortable to users who are physically challenged;

- the library shall provide space for the use of patron's personal equipment

- the library shall provide internet facilities for the use of students;

- the library shall provide scanning and printing facilities for students;

- from a functional point of view the minimum ceiling heights of various user space in the library shall conform to the specifications contained in the following Table 3-

Table 3 – Minimum Ceiling Heights

| Functional Area | Floor Area | Minimum Ceiling Heights |
| --- | --- | --- |
| Reading area | Less than 10 square metres | 2.3 metres |
| Reading Area | Greater than 10 square metres | 2.7 metres |
| Stack Area |  | 2.3 metres |

- Minimum space requirements of a tertiary library

Minimum space requirements of a tertiary library

| ITEM | DESCRIPTION | SPACE IN SQUARE METRES PER PERSON |
| --- | --- | --- |
| Reader work space/carrels |  | 3.0 |
| 1 Computer work stations in smaller Institutions |  | 4.0 |
| 2 Computer work stations in larger Institutions |  | 3.25 |
| Space for seat in an instruction room |  | 3.0 |
| Space for seat in conference room |  | 3.0 |
| Space for seat in lecture in lecture hall |  | 1.0 |
| Library staff work area | To contain technical services | 12.50 in smaller institution or 10.00 in larger institution |
| Multimedia carrels |  | 4.0 |
| Photocopier | Provision for ventilation, storage counter, space for book trolley and bins | 6.0 |
| Shelving space | For every 100 volumes to be housed in regular shelving. Therefore space for stock of 2000 is divided by 100 to arrive at 20 square metres. | 1.0 |

Minimum dimensions of reading tables

| Description | Dimension |
| --- | --- |
| Table to sit one person | 90cm x 60cm |
| Table to sit two persons | 90cm x 120cm |
| Table to sit four persons | 180cm x 120cm |
| Table to sit six persons | 250cm x 120cm |

Auditorium/Large Lecture Theatre/Assembly Hall

Lecture halls are places of social and personal interaction, where learning takes place and where

creative thinking is encouraged. A successful layout for an auditorium must achieve the best possible combination of architectural elements and teaching facilities so that both teaching and learning is maximized. The facility shall be designed such that audio visual aids can be used to enhance teaching.

- A tertiary institution may provide a building facility for public lectures or general assembly which shall be in form of an auditorium, a large lecture theatre or an assembly hall conforming to the following specifications-

Table 4 – Minimum Space per seat in Auditorium, Lecture Theatre and Assembly Hall

No. of Seats				Assignabe space per seat in square metres

60-100	..	..	..	..			0.9

101-150	..	..	..	..			0.8

151-300	..	..	..	..			0.7

301-500	..	..	..	..			0.6

Above 500							0.5

- the assembly hall or auditorium shall conform to the requirements of the National Building Regulations LI 1630

- The hall shall be designed so as to provide clear view of the stage.

- In an existing building with a flat floor, flat screens/monitors shall be so placed to afford the audience a clear view.

- As far as possible  columns shall be placed so as not to obstruct sight lines

- Where tablets have been provided at least 10% of them shall be left- handed,

- Doors shall open outwards so as to permit quick evacuation from the auditorium,

- A sufficient number of doors shall be provided for a maximum clearance time of 5 minutes for quick and efficient changeover between lectures with at least one door at or near rear of lecture theatre for the entry of late comers;

- The front row of seats shall be at the same level as the adjacent entry doors for disabled persons access

- Wheel chair spaces are to be located towards the middle of the front row rather than the side

- Allow at least 1 wheel chair space for every 100 seats

PART V – TEACHING LABORATORIES, HOSPITALS, WORKSHOPS AND STUDIOS

- A tertiary institution with the intention to run or running practical-related programmes shall, in addition to providing classrooms and lecture theatres provide adequate laboratory/workshop facilities.

- In assessing the adequacy of laboratory facilities the following shall be taken into account-

- the provision of adequate laboratory facilities in close proximity to classroom or other teaching facilities;

- the provision of room for instructional workstations and each workstation per student conforming, where possible to the specifications contained in the following Table 5-

Table 5- Laboratory Assignable Space per student

Laboratory Specialty	Assignable space per Student in square metres

Agriculture Science Laboratory		2.80-3.70

Building Science Laboratory		3.70-4.60

Biological Health and Veterinary Science Laboratory 		2.70-4.60

Business Management Laboratory 		1.90-2.80

Communication Laboratory		2.40-3.70

Education Laboratory 		2.40-4.60

Engineering Laboratory (excluding mechanical)		3.70-6.50

Engineering Laboratory (Mechanical)		6.50-9.30

Fine Art, Architecture Design Studio		2.80-4.60

Home Economics Laboratory		2.80-4.60

Computing laboratory		2.80-3.70

- the provision of storage and preparatory rooms and a laboratory office;

- proper location in relation to other facilities such that the laboratory is easily accessible;

- the provision of the following service facilities-

- adequate windows which open to outside air in order to provide proper ventilation,

- fume cupboards and fume hoods in the preparation and storage rooms,

- standard table top electrical service outlets for the instructor’s table and for each pair of students workstation,

- adequate lighting free of flare from exterior sunlight,

- surface treatment of floors, walls and ceiling to reduce noise,

- a sink with hot and cold water service installation for the instructor’s table and for each pair of students workstation.

- gas for the instructor’s table and for each pair of students workstation,

- the provision of the built-in furniture and equipment –

- at least one instructor’s table measuring 2000 x 300 x 760 mm high,

ii.	at least one student table of minimum size 1500 x 560 x 760mm, high for each pair of students workstation with adequate bench or stools or chairs,

iii.	chalkboard or blackboard, wall mounted, a white board/projection screen

iv.	at least one corrosion resistant sink and eye-wash facility.

- A tertiary institution offering medical courses shall have the necessary basic pre-clinical sciences together with the laboratory requirements specified in paragraph 24 to 25 plus any specialized provisions for anatomy and pathology; and the tertiary institution shall also have or secure access to a teaching hospital easily accessible to students and equipped with the usual outpatient and inpatient facilities covering all specializations in medicine, surgery, obstetrics and gynaecology, paediatrics, community health, as well as support facilities such as X-ray, radiotherapy, anaesthetics and pathology, accessible to medical students;

Provided that where the hospital is not the property of the relevant tertiary institution, adequate legal and administration arrangements shall be made in respect of such access.

- The hospital shall have the necessary resources (human, physical, technical and financial) to meet professional and academic requirements.

- The design of the teaching hospital shall conform to the requirements of the Ministry of Health in providing relevant health services; and in addition, the hospital buildings shall conform to the requirements of the National Building Regulations.

- A tertiary institution offering Fine Arts shall have a studio for drawing from life, general drawing, basic design painting, ceramic and film room; and it shall also have graphic photography print making studio, life painting, composition/still life studios, textile design and weaving, and sculpture studios and the tertiary institution shall also have –

- a ceramic glazing and testing laboratory, glass design and technology, silver smiting and jewelry and museology laboratory;

- workshop space for modeling casting and woodcarving;

- space for sculpture, garden, outdoor wood burning kilns and such other facilities as may be specified by the Commission;

- studio spaces and drafting room with adequate lighting;

- photogrammetic laboratory;

- remote sensing laboratory;

- computing laboratory;

- hydrographic laboratory workshop;

- cartographic workshop;

- photographic laboratory or workshop.

- A tertiary institution offering Urban and Regional Planning shall have adequate studio space for every year of study and the tertiary institution shall also have a model making workshop.

- A tertiary institution offering Building or Building Technology shall have studio space for every year of study and the tertiary institution shall also have: -

- for every year of study, workshops for carpentry and joinery, mechanical works and welding, masonry and concrete works and electrical wiring and building services;

- laboratory space for each course of study for materials sciences, structures and services;

- spaces for demonstration units in various building construction techniques.

- The facilities specified in paragraphs 29 and 31 may be shared between years of study and between courses or they may be assigned on the basis of faculties.

- A tertiary institution offering Architecture shall have design studios for each class with adequate individual workshop space at the rate of 1.5 square metres per work station and the tertiary institution shall also have-

- a modeling workshop with equipment enough for at least two class years;

- an architectural science laboratory with sufficient equipment;

- a photographic laboratory;

- one material testing workshop;

- one wood working workshop;

- space within the overall premises of the department for external activities such as those required for workshops  fabrication and for erection of building components and for testing their exposure to  weather activities.

- A tertiary institution offering Home Economics shall have laboratories consisting of the following user areas:-

  - food analysis to the extent of 5 square metres per student workstation.

  - food storage and preparation to the extent of 1.5 square metres per student workstation;

  - sewing and crafts to the extent of 4 square metres per student workstation;

  - sewing and craft storage and dressing to the extent of one square metre per student workstation;

  - child development with its own storage room, office, open play, cleaning up and observation rooms.

- Every food laboratory shall be provided with installations adequate for instructions in food preparation, cooking and services using a variety of cooking energy sources; electricity, gas and wood.  Home care laboratories shall also be equipped with laundry and ironing facilities;

- A tertiary institution offering Agriculture shall, in addition to the laboratory specifications specified in paragraph 34; have at least ten hectares of land set aside as a tertiary institution farm over and above that stipulated for an ordinary tertiary institution.

PART VI – CATERING AND ACCOMMODATION

- A residential tertiary institution shall provide adequate, eating facilities that meet the standards of the regulatory bodies as defined by the Ghana Tourist Board and the Ministry of Health and its agencies.

- Where students have to do their own cooking, facilities for self-catering must be provided

- In assessing the adequacy of these facilities the following factors may serve as a guide

  - the provision of the following functional areas: -

    - food preparation of not less than 0.50 square metre per student

    - dish washing and serving areas, of not less than 0.50 square metre per student;

    - kitchen storage of not less than 0.50 square metre per student;

    - kitchen staff and services area of not less than 0.50 square metre per student

    - a dining hall of not less than 1.25 square metre of floor area per student;

    - left over food and refuse transfer area which should not allow contamination of fresh food, cooked or uncooked food,

  - the provision of adequate facilities for cleaning utensils, and of suitable and sufficient washing and sanitary facilities in form of lavatories, shower rooms, changing and cloakrooms for kitchen staff;

- Every tertiary institution shall provide common rooms with adequate recreational facilities for staff and students.

- Every residential tertiary institution shall provide adequately lighted and ventilated student accommodation facilities and in particular-

  - no open rooms used as student accommodation shall accommodate more than four students;

  - the total floor area of the rooms shall not be less than the following per student

    - single rooms, 8.00 square metres;

    - double student room or cubicle, 7.00 square metres;

    - three or four student room 6.00 square metres;

  - every living room in the hall shall be located so as to get maximum benefit of outside light and air;

  - every room or hall which does not have air conditioning shall be provided with vents in the walls in such a manner as to provide permanent ventilation;

  - sufficient doorways shall be provided in every student hall to ensure rapid exit to ease of fire or other emergencies;

  - adequate wardrobe space shall be provided for hanging clothes. The minimum clear depth is 600mm and width should not be less than 600mm per student;

  - at least three number 13 amps socket shall be provided per student in each room;

PART VII – UTILITY SERVICES

- Every tertiary institution shall have adequate water supply installed in accordance with applicable requirements of the National Building Regulations, the Ghana Water Company and the National Standards Board.

- In assessing the adequacy of the water supply for the tertiary institution the following shall also be taken into account:

  - the extent to which the quality of the water provided conforms to the requirements of the National Drinking Water Standards set out in the latest edition of the Water Supply Design Manual issued by the Ghana Water Company;

  - the availability of  water storage  facility in each building for domestic purposes taken from the main supply to the buildings or a water storage tank in each building with sufficient capacity to meet not less than seventy two hours demand for the building’s users or a minimum water demand of 150 litres per day per person;

  - the  availability of a central water storage facility from the main water supply to the tertiary institution or of a storage facility with sufficient capacity to meet not less than three days demand from the tertiary institution community or a minimum water demand of 150 litres per day per person;

  - the availability of extra additional storage facility under sub-paragraph(b) in every tertiary institution where animals are kept;

  - the provision and maintenance of an adequate water storage facility for firefighting purposes at the rate of10 litres per second continuous flow for two hours.

- Every tertiary institution shall provide appropriate and adequate waste water and sewage management system as certified by the appropriate regulatory body.

- In assessing the adequacy of the waste water system the following shall be taken into account-

  - the availability of a satisfactory drainage system for water;

  - the availability of an adequate means of sewage and waste water disposal by means either or the relevant  local authority main sewer or of septic tanks, conservancy tanks of stabilization ponds or cesspool constructed to specifications contained in the National Building Regulations;

  - in the absence of adequate means of sewage and waste disposal the extent to which the quality of effluent of natural water bodies conform to the National Standards for Effluent Discharge of Natural Streams;

  - evidence of approval by the relevant local authority or Ministry of Works or the Ministry of Health for all disposal systems other than those connected to a local authority’s sewage system.

- Every tertiary institution shall provide, keep clean and maintain adequate and suitable sanitary conveniences, which conform in all respects to the requirements of the Building Code, for the students and all persons working in the tertiary institution and where students or other persons of both sexes are accommodated or are expected to be accommodated the conveniences shall afford proper separate facilities for persons of each sex.

- In a non-residential tertiary institution, the following well-lit and ventilated closets, urinals, wash hand basins and drinking water fountains shall be provided:

| FEMALE STUDENTS |  |  | MALE STUDENTS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ### No of Girls | No of WCs | No of Wash hand basins | No of Boys | No of WCs | No of Urinals | No of Wash hand basins |
| 1-10 | 2 | 1 | 1-10 | 1 | 1 | 1 |
| 11-20 | 2 | 2 | 11-20 | 1 | 2 | ### 2 |
| 21-25 | 2 | 2 | 21-25 | 2 | 2 | 2 |
| 26-50 | ### 3 | 2 | 26-50 | 2 | 2 | 2 |
| 51-75 | 4 | 3 | 51-75 | 3 | 3 | 3 |
| 76-100 | 5 | 3 | 76-100 | 3 | 3 | 3 |
| 101-125 | 6 | 3 | ### 101-125 | 4 | 3 | 3 |
| 126-150 | 6 | 4 | 126-150 | 4 | 3 | 4 |
| 151-175 | 7 | 4 | 151-175 | 4 | 4 | 4 |
| 176-200 | 7 | 4 | 176-200 | 4 | ### 4 | 4 |
| 201-225 | ### 8 | 5 | 201-225 | 5 | 4 | 5 |
| 226-250 | 8 | 5 | 226-250 | 5 | 4 | 5 |
| 251-275 | 9 | 5 | 251-275 | 5 | 5 | 5 |
| 276-300 | 9 | 6 | 276-300 | 5 | 5 | ### 6 |
| 301-325 | 10 | ### 6 | 301-325 | 6 | 5 | 6 |
| 326-350 | 10 | 6 | 326-350 | 6 | 5 | 6 |
| 351-375 | 11 | 7 | 351-375 | 6 | 6 | 7 |
| 376-400 | 11 | 7 | 376-400 | 6 | 6 | 7 |
| ### 401-425 | 12 | 7 | 401-425 | 7 | 6 | 7 |
| 426-450 | 12 | 8 | 426-450 | 7 | 6 | 8 |
| 451-475 | 12 | 8 | 451-475 | 7 | 7 | 8 |
| 476-500 | 13 | 8 | 476-500 | 7 | ### 7 | 8 |
| 501-525 | ### 14 | 9 | 501-525 | 8 | 7 | 9 |
| 526-550 | 14 | 9 | 526-550 | 8 | 7 | 9 |
| 551-575 | 15 | 9 | 551-575 | 8 | 8 | 9 |
| 576-600 | 15 | 10 | 576-600 | 8 | 8 | 10 |
| Above 600 | Add 1 WC for every 35 students | Add 1 Wash hand basin for every 60 students | Above 600 | Add 1 WC for every 75 students | Add 1 Urinal for every 75 students | Add 1 Wash hand basin for every 60 students |

- In a hostel for a tertiary institution, the following well lighted and ventilated closets, urinals, wash hand basins, and bathrooms shall be provided-

  - one water closet for every 8 students;

  - one wash hand basin per every 8 students;

  - one bath or shower per every 8 students;

PART VIII – LAND REQUIREMENTS

- Every tertiary institution shall produce evidence of legal occupancy of at least 5 years with a provision for renewal.

- Where the institution owns the land it shall show evidence of owning land on freehold terms, or for a leasehold term of not less than 30 years; and the land shall, in addition –

  - be free from all encumbrances other than those of a statutory nature;

  - be vested in a trusteeship of the tertiary institution, which shall own such land as non-profit making body and in perpetual succession.

- All tertiary institution buildings and facilities thereon shall conform to the minimum space and compound requirements stipulated in these Standards, the National Building Regulations, the Ministry of Health Act or any other legislation relating to the safeguarding of public safety and educational standards.

- Land at the main campus and at all the other non-contiguous parcels shall be used for tertiary institution purposes, and for such other uses as are necessary for supporting tertiary institution activities; and the use shall, in addition, conform to master plan (s) prepared in accordance with these Standards and all applicable laws and designed in such a way that all building facilities and proposed developments are functionally related and compatible.

- The master plan(s) shall, in addition to any other details required by law, include and indicate-

  - the location of the existing and proposed tertiary institution buildings;

  - the layout of all roads and pathways serving all buildings which shall be such that the health and safety of the occupants and users of the tertiary institution is not endangered;

  - the layout of all waste and soil drain pipes, sewers, culverts, septic tanks, storm water drains, and run-off catchment drains so laid that the health and safety of all occupants and users of the tertiary institution is not endangered;

  - the location of all areas to be used by all other activities related to the tertiary institution such as sports fields, tertiary institution farms, forests, arboreta and sewage treatment;

  - the layout of all utility service lines which shall be laid in a safe manner.

- The minimum total area set aside for open space shall conform to the regulations in force.  The space allocated for parking of cars expected to park on the campus shall conform to the recommendations of the Traffic Impact Assessment Report as accepted and approved by the Department of Urban Roads and the Environmental Protection Agency. This area shall not incorporate land set aside for sporting facilities.

- A tertiary institution shall have access to at least one standard recreational facility.